Faculty of Information Engineering and Technology

Signals and Systems Theory — COMM 401

Prof. Ahmed El Mahdy

Eng.Reem

Audio Matching Using

Fourier Transform (FFT)

Project Report

Submitted by:

Sama Ismael

ID: 64-19880

# 1. Python Code

import numpy as np

import matplotlib.pyplot as plt

from scipy.io import wavfile

def read_audio_file(filename):

sampling_rate, audio_signal = wavfile.read(filename)

if audio_signal.ndim > 1:

audio_signal = np.mean(audio_signal, axis=1)

audio_signal = audio_signal.astype(np.float32)

return sampling_rate, audio_signal

def calculate_fourier_magnitude(segment):

segment = segment * np.hamming(len(segment))

fourier_result = np.fft.rfft(segment)

magnitude = np.abs(fourier_result)

return magnitude

def main():

audio_filename = 'quran.wav'

start_time_seconds = 15.0

clip_duration_seconds = 3.0

step_size_seconds = 0.1

sample_rate, full_signal = read_audio_file(audio_filename)

full_time_axis = np.arange(len(full_signal)) / sample_rate

start_index = int(start_time_seconds * sample_rate)

duration_in_samples = int(clip_duration_seconds * sample_rate)

end_index = start_index + duration_in_samples

query_clip = full_signal[start_index:end_index]

clip_time_axis = np.arange(len(query_clip)) / sample_rate

clip_magnitude = calculate_fourier_magnitude(query_clip)

frequency_axis = np.fft.rfftfreq(len(query_clip), 1 / sample_rate)

step_samples = int(step_size_seconds * sample_rate)

window_length = len(query_clip)

similarity_scores = []

time_steps = []

clip_normalization = np.linalg.norm(clip_magnitude)

for current_index in range(0, len(full_signal) - window_length + 1, step_samples):

current_window = full_signal[current_index:current_index + window_length]

window_magnitude = calculate_fourier_magnitude(current_window)

window_normalization = np.linalg.norm(window_magnitude)

if clip_normalization == 0 or window_normalization == 0:

similarity = 0

else:

similarity = np.dot(clip_magnitude, window_magnitude) / \

(clip_normalization * window_normalization)

similarity_scores.append(similarity)

time_steps.append(current_index / sample_rate)

best_index = np.argmax(similarity_scores)

detected_time = time_steps[best_index]

highest_score = similarity_scores[best_index]

detected_sample = int(detected_time * sample_rate)

matched_signal = full_signal[detected_sample:detected_sample + window_length]

print('Sampling frequency:', sample_rate, 'Hz')

print('Detected position:', detected_time, 'seconds')

print('Best similarity score:', highest_score)

if __name__ == '__main__':

main()

# 2. Results

## 2.1 Printed Output

| Parameter | Value |
| --- | --- |
| Sampling Frequency | 44,100 Hz |
| Length of Full Signal | 891.46 seconds |
| Clip Length | 3.00 seconds |
| Original Clip Position | 15.00 seconds |
| Detected Position | 15.00 seconds |
| Best Similarity Score | 1.0000 |

# 3. Signal Plots

Figure 1 — All Plots (Combined View)

Figure 2 — Full Signal (Time Domain)

Figure 3 — Query Clip (Time Domain)

Figure 4 — Query Clip (Frequency Domain — FFT)

Figure 5 — Similarity Score vs. Time

Figure 6 — Original Clip vs. Detected Segment